## Remote board Firmware Development Record

Project Name: EG4 6K

| No. | Firmware version | Solution | Date/Author |
| --- | --- | --- | --- |
|  | 61.00 | 1. Add the AC coupling setting item. 2. Add EG4 BMS communication. |  |
|  | 61.02 | To optimize the BMS communication exception problem, refer to the 6.5KW solution. |  |
|  | 61.03 | Troubleshoot the fault that some alarms cannot be displayed because the alarm judgment logic is abnormal |  |
| 1 | 61.05 | 1. Modify BMS warning logic 2. Fix parallel setting display | 2023.02.09/Mingjian |
| 2 | 61.07 | 1. Fix setting revert (61.06 sent to TPE lab for testing)) | 2023.02.11/Mingjian |
| 3 | 61.08 | 1. Optimize BMS communication. Fix that when SOC is less than 20%, the buzzer alarms but the LCD has no alarm code display. 2. Fix that after the Zetara battery is updated to the latest version of the agreement, the inverter protects itself before the BMS present low-voltage protection | 2023.02.15/Mingjian |
| 4 | 61.11 | Solve the problem reported by customers that they can’t install old version firmware (cannot be updated from 61.10 to 61.08) | 2023.03.28/Mingjian |
| 5 | 61.12 | Modify the warning32 occurrence conditions. Add the prevention mechanism, and reduce false alarms caused by communication interference | 2023.03.29/Mingjian |
| 6 | 61.13 | Optimize the communication logic between the control board and the communication board to solve the problem of abnormal communication. |  |
| 7 | 61.14 | Solves problem that SOC setting cannot be saved when the machine is in parallel |  |
| 8 | 61.15 | 1. Add setting item 34 to control N-G relay state (N-G RELAY CLOSE) 2. Add command ^P006PSn to improve parallel data display |  |

## DSP Firmware Development Record

Project Name: EG4 6K

| No. | Firmware version | Solution | Date/Author |
| --- | --- | --- | --- |
| 1 | 63.04 | Increase PV operating voltage range according to customer requirements: 450V->500V | 2023.02.09/Mingjian |
| 2 | 63.05 | Added the AC coupling function | 2023.02.11/Mingjian |
| 3 | 63.06 | The AC coupling function is improved, and the system stability is improved by adding SOC control logic. | 2023.02.15/Mingjian |
| 4 | 63.07 | Increase single-phase unbalanced load capacity: 3kw->4.5 kw | 2023.03.28/Mingjian |
| 5 | 63.08-63.09 | The problem of changing the setting item is solved by limiting the writing condition to eeprom. | 2023.03.29/Mingjian |
| 6 | 63.13 | 1. Optimized the MPPT controller, optimized the flow sharing control algorithm to solve the problem of customer feedback output jitter. 2. Modify the control logic of the ngrid-relay to power on the ngrid-relay to ensure that network n and OP-N are always connected. |  |
| 7 | 63.14 | Add SOC controls. |  |
| 8 | 63.15 | Improved the logic of alarm 04 caused by SOC. |  |
| 9 | 63.19 | Modified MPPT controller to solve customer MPPT tracking card in low power situation. |  |
| 10 | 63.20 | The eeprom initialization reading scheme is optimized by referring to 6.5kw logic. |  |
| 11 | 63.21 | According to the customer's feedback, setting item 19 could not be saved in parallel mode. It was found that when adding SOC control items, SOC parameter synchronization data was incorrectly saved, so it was corrected. |  |
| 12 | 63.22 | # Solve uneven output current when PV and Battery are loaded |  |
| 13 | 63.23 | 1. Add N-G relay control logic. The setting will only effect when machine enters the standby mode. In other words, after changing the setting, machine needs to enter the standby mode, and relay would take action. a. Set to disable, N-G relay is disconnected, output N and mains N are turned on. b. Set to enable, the output N and the grid N are turned on in the grid mode, and N-G is disconnected. In other modes, the N-G is turned on, and the output N and the grid N are disconnected. 2. Add command ^P006PSn to improve parallel data display |  |
| 14 | 63.24 | # Correct the Disable/Enable N-G Selection order |  |